# ENVIRONMENTAL & COMPLIANCE PROCEDURES

ENVIRONMENTAL & COMPLIANCE PROCEDURES	1

1. SOPEP / VESSEL RESPONSE PLAN (VRP) PROCEDURE	2

1. PURPOSE:	2

2. RESPONSIBILITY:	2

3. PROCEDURE:	2

3.1 Immediate Actions (First 15 Minutes):	2

3.2 Containment & Mitigation:	2

3.3 Coordination & Communication:	2

3.4 Clean-up & Recovery:	2

3.5 Post-Incident Actions:	2

4. REFERENCES:	2

2. GARBAGE MANAGEMENT PLAN PROCEDURE	3

1. PURPOSE:	3

2. RESPONSIBILITY:	3

3. PROCEDURE:	3

3.1 Segregation at Source:	3

3.2 Processing & Storage:	3

3.3 Discharge Operations:	3

3.4 Documentation:	3

3.5 Crew Training & Placards:	3

4. REFERENCES:	3

3. BALLAST WATER MANAGEMENT PLAN (BWMP) PROCEDURE	4

1. PURPOSE:	4

2. RESPONSIBILITY:	4

3. PROCEDURE:	4

3.1 Plan Familiarity & Local Laws:	4

3.3 Ballasting Operations (Taking on Ballast):	4

3.4 Deballasting Operations (Discharging Ballast):	4

3.5 Sediment Management:	4

3.6 Reporting:	4

4. REFERENCES:	4

4. ANTIFOULING PAINT PROCEDURE	5

1. PURPOSE:	5

2. RESPONSIBILITY:	5

3. PROCEDURE:	5

3.1 Pre-Docking Planning:	5

3.2 Safe Application (During Drydocking):	5

3.3 In-Water Cleaning & Maintenance:	5

3.4 Documentation & Certification:	5

4. REFERENCES:	5

5. REPORTING PROCEDURE FOR INCIDENTS & NON-CONFORMITIES	6

1. PURPOSE:	6

2. RESPONSIBILITY:	6

3. PROCEDURE:	6

3.1 What Must Be Reported (Immediately):	6

3.2 Immediate Actions (On-Scene):	6

3.3 Initial Reporting (Within 1 Hour for Serious Events):	6

3.4 Investigation & Root Cause Analysis:	6

3.5 CAPA & Close-Out:	6

All reports and investigations are filed in the vessel's SMS records.	6

4. REFERENCES:	6

# 1. SOPEP / VESSEL RESPONSE PLAN (VRP) PROCEDURE

## 1. PURPOSE:

To provide a structured framework for responding to an oil pollution incident, minimizing environmental impact, and ensuring compliance with MARPOL and regional regulations (e.g., US VRP).

## 2. RESPONSIBILITY:

Master (Incident Commander), Chief Officer (On-Scene Coordinator - Deck), Chief Engineer (On-Scene Coordinator - Engine), All Crew.

## 3. PROCEDURE:

### 3.1 Immediate Actions (First 15 Minutes):

STOP THE SOURCE: Priority #1. This may involve:

Deck Spill: Shutting valves, transferring cargo/fuel, patching leaks.

Engine Room Spill: Isolating pumps, closing scuppers, activating emergency shutdowns.

RAISE ALARM: Sound general alarm. Announce "OIL SPILL" with location over PA/VHF.

ASSESS & REPORT: Determine spill type (fuel, lube, cargo), rate, and source.

INITIAL NOTIFICATION: Master or designate makes immediate mandatory reports via fastest means (VHF, Sat-C):

Flag State (as per SOPEP contact list).

Coastal State (nearest Port Authority/RCC).

Company Designated Person Ashore (DPA) – 24/7 contact.

### 3.2 Containment & Mitigation:

Deploy Spill Kit: Crew trained in spill response deplates:

Absorbent pads/booms around source and overboard scuppers.

Containment booms around vessel if practical and safe.

Prevent Overboard Spread: Close all non-essential overboard discharges, scupper plugs, and sea chests.

Minimize Hazard: Eliminate ignition sources (no smoking, hot work). Ventilate enclosed spaces if vapors present.

### 3.3 Coordination & Communication:

Establish Command Center: Typically the bridge. Use SOPEP/VRP Flowcharts for decision-making.

Maintain Log: Chronological record of all actions, communications, and observations.

Activate Shore Support: DPA mobilizes company resources and qualified spill response contractor as required by plan.

### 3.4 Clean-up & Recovery:

Follow Contractor Lead: Once professional responders are on scene, vessel crew supports as directed.

Internal Clean-up: Use absorbents, pumps, and rags. Collect all waste in marked drums for proper disposal.

Monitor Weather: Forecast may impact response strategy (wind, currents).

### 3.5 Post-Incident Actions:

Preserve Evidence: Do not clean up or repair spill source until authorized (for investigation).

Detailed Reports: Master submits written report to Flag State, Coastal State, and Company within required timeframe.

Review & Drill: Conduct debrief. Update SOPEP/VRP based on lessons learned. Increase frequency of related drills.

## 4. REFERENCES:

MARPOL Annex I, Reg 37, IMO Resolution MEPC.54(33) (SOPEP Guidelines), US Coast Guard 33 CFR 155 (VRP).

# 2. GARBAGE MANAGEMENT PLAN PROCEDURE

## 1. PURPOSE:

To manage, process, and dispose of all garbage in compliance with MARPOL Annex V to prevent pollution.

## 2. RESPONSIBILITY:

Chief Engineer (overall), Chief Officer (deck garbage), Chief Steward (galley/domestic), all crew.

## 3. PROCEDURE:

### 3.1 Segregation at Source:

Garbage is separated into color-coded streams at dedicated stations:

Plastic (Red): ALL plastics, including bags, wrappers, bottles. NEVER discharged at sea.

Food Waste (Green): Committed only.

Metal/Glass/Cardboard (Blue): Cans, bottles, packaging.

Incinerator Ash (Black): From ship's incinerator.

Operational Waste (Orange): Fishing gear, cleaning agents.

### 3.2 Processing & Storage:

Food Waste: May be discharged > 3nm from land (ground to <25mm if >12nm). Otherwise stored for port discharge.

Plastic, Ash, & Oily Rags: Must be stored for discharge to port reception facilities only.

Metal/Glass: May be discharged > 25nm from land. Often compacted and stored for recycling ashore.

Incinerator: May be used for paper/cardboard/food waste when > 3nm from land. Plastics must NOT be incinerated on board (can produce toxic fumes). Ash must be retained.

### 3.3 Discharge Operations:

At Sea: Only when vessel is proceeding and in permitted areas as per MARPOL Annex V Discharge Tables posted at garbage stations.

To Shore: Contact agent 24 hours before arrival to request garbage disposal. Complete Port Waste Reception Form.

Witness & Record: A ship's officer witnesses shore discharge and obtains a dated receipt.

### 3.4 Documentation:

Every discharge (to sea or shore) is recorded in the Garbage Record Book.

Entries include: Date/time, position, category/estimated amount of garbage, method of discharge. Signed by officer in charge.

Receipts from shore facilities are pasted into the book. The book must be retained on board for 2 years.

### 3.5 Crew Training & Placards:

All crew are trained on the plan during induction.

MARPOL Annex V Placards (detailing discharge rules) are posted in conspicuous areas (mess, galley, dock landings).

## 4. REFERENCES:

MARPOL Annex V, IMO Resolution MEPC.295(71) (2016 Guidelines), Company Garbage Management Plan.

# 3. BALLAST WATER MANAGEMENT PLAN (BWMP) PROCEDURE

## 1. PURPOSE:

To manage ballast water and sediments to prevent the spread of invasive aquatic species, in compliance with the BWM Convention and local regulations.

## 2. RESPONSIBILITY:

Chief Officer (Ballast Water Management Officer - BWMO), OOW (operations), Chief Engineer (system maintenance).

## 3. PROCEDURE:

### 3.1 Plan Familiarity & Local Laws:

BWMO must know the vessel's specific Ballast Water Management Plan and Record Book.

Critical: Check local/port state regulations for each port of call. Requirements can be stricter than IMO D-2 standard (e.g., US Coast Guard).

3.2 Management Options (Per Plan):
The vessel must use at least one of the following methods for each ballast operation:

D-2 Standard (Treatment): Use the installed Ballast Water Treatment System (BWTS). Operate strictly per manufacturer's manual. Maintain treatment log.

Ballast Water Exchange (D-1 Standard):

Sequential Method: Empty and refill tank at sea (>200nm from shore, >200m depth).

Flow-Through Method: Pump sea water through tank at 3x tank volume.

Only permitted in designated areas (≥200nm from land and ≥200m depth).

Discharge to Shore Reception: If available and agreed with port authorities.

### 3.3 Ballasting Operations (Taking on Ballast):

Avoid uptake in areas: with known outbreaks/spores, near sewage outfalls, in shallow/ turbid water, at night (when organisms may rise).

Record: In Ballast Water Record Book – start time, position, tank, volume, salinity.

### 3.4 Deballasting Operations (Discharging Ballast):

Confirm treatment/exchange has been completed as required for destination port.

Sample & Test: Be prepared for Port State Control inspectors to take samples. Ensure treatment system was operational and records are perfect.

Record: In Ballast Water Record Book – start time, position, tank, volume, method used (D-1 or D-2), salinity.

### 3.5 Sediment Management:

Bottom of Tanks: Remove accumulated sediments in designated cleaning areas (≥200nm from shore, ≥200m depth) or to shore reception facilities.

Record sediment removal/disposal in the Record Book.

### 3.6 Reporting:

Provide Ballast Water Reporting Form to port authorities as required (typically 24-48 hours before arrival).

## 4. REFERENCES:

IMO BWM Convention (2004), BWMP specific to vessel, USCG Ballast Water Regulations (33 CFR 151).

# 4. ANTIFOULING PAINT PROCEDURE

## 1. PURPOSE:

To manage the application and maintenance of antifouling systems in compliance with the AFS Convention (ICAFS), preventing the transfer of harmful organisms and using biocides safely.

## 2. RESPONSIBILITY:

Chief Officer (planning/application oversight), Bosun (execution), Master & Company (vendor/contractor selection).

## 3. PROCEDURE:

### 3.1 Pre-Docking Planning:

Select AFS Type: In consultation with Company Superintendent, select paint system compatible with hull substrate and previous coatings. Consider operational profile (speed, trading area).

Review ICAFS Certificate: Ensure new system complies. Tributyltin (TBT)-based paints are PROHIBITED.

Contractor/Vendor Selection: Use approved, environmentally certified contractors. Ensure Material Safety Data Sheets (MSDS) for all products.

### 3.2 Safe Application (During Drydocking):

Containment: All blasting and painting must occur within a sealed containment area to capture dust and paint chips.

Waste Management: All waste is hazardous. Collect and store for disposal as Special Waste by licensed contractor. Never wash into dock or sea.

Crew Safety: Applicators must wear full PPE – respirators, coveralls, gloves, eye protection. Restrict access to area.

Surface Preparation: Hull must be prepared per paint manufacturer's specifications (e.g., blast profile, cleanliness).

### 3.3 In-Water Cleaning & Maintenance:

Prohibition: In-water cleaning of AFS that release biocides is strictly prohibited unless using a capture system approved by local authorities.

Soft Cleaning: Only gentle wiping to remove slime (not removing paint) is permitted without capture.

Approval Required: Any in-water cleaning that removes paint must have prior port state and environmental authority approval.

### 3.4 Documentation & Certification:

International Antifouling System Certificate (IAFSC): Must be on board. Updated after each new application.

Paint Manufacturer's Declaration: Confirming paint does not contain controlled biocides (Cybutryne/Seanine 1051 is now prohibited as of 2023).

Waste Disposal Receipts: Retain all receipts for hazardous waste disposal from the yard.

## 4. REFERENCES:

IMO AFS Convention (ICAFS), IMO Resolution MEPC.331(76) (2021 Guidelines), Manufacturer's Specifications.

# 5. REPORTING PROCEDURE FOR INCIDENTS & NON-CONFORMITIES

## 1. PURPOSE:

To ensure all accidents, near-misses, hazardous occurrences, and breaches of SMS/procedures are reported, investigated, and acted upon to prevent recurrence (ISM Code 9).

## 2. RESPONSIBILITY:

All Crew (to report), Head of Department (initial investigation), Master (serious incidents), Company DPA (review & analysis).

## 3. PROCEDURE:

### 3.1 What Must Be Reported (Immediately):

Very Serious: Death, serious injury, loss of person overboard, collision, grounding, fire, explosion, structural failure, pollution.

Serious: Less severe injury, machinery failure affecting safety, navigation incident, security breach, breach of alcohol policy.

Near-Miss/Hazardous Occurrence: Any event that under slightly different circumstances could have led to serious consequences (e.g., dropped object, slip/trip, miscommunication).

Non-Conformity: Any observed failure to follow the SMS, procedures, or regulations.

### 3.2 Immediate Actions (On-Scene):

Attend to safety: Secure area, provide first aid, mitigate immediate danger.

Preserve Evidence: Do not disturb scene if possible. Take photos.

Notify Bridge/Master immediately.

### 3.3 Initial Reporting (Within 1 Hour for Serious Events):

Master reports to Company DPA via designated 24/7 contact.

DPA guides on mandatory external reporting to Flag State, Class, or coastal authorities.

Initial Report Form completed with: What, When, Where, Who involved, immediate actions taken.

### 3.4 Investigation & Root Cause Analysis:

Master appoints Investigation Team (typically Sr. Officers not directly involved).

Team uses 5-Whys or Fishbone Diagram to find root cause, not just blame.

Interviews personnel, reviews records, examines equipment.

Produces Investigation Report with: Timeline, root causes, and Corrective/Preventive Action (CAPA) recommendations.

### 3.5 CAPA & Close-Out:

DPA/Company reviews report, approves CAPA.

Assigned Owner implements CAPA (e.g., procedure change, training, repair).

Master verifies implementation is effective.

Case is closed in the Non-Conformity/Incident Tracking System.

3.6 Records & Trends:

### All reports and investigations are filed in the vessel's SMS records.

Company analyzes trends across the fleet to identify systemic issues.

Lessons are shared via safety alerts or SMS updates.

## 4. REFERENCES:

ISM Code Sections 9 & 12, Flag State Reporting Requirements, Company SMS Chapter 9.